# End of Manufacturing Tool Developer Manual

# About the tool

End of Manufacturing Tool (EoM) is designed to check and set system settings.

# Important Notes

To release your dll in EoM tool, please run Palamida scan first. Then send an email to dl.bios.verification, containing the dll you want to release.

# Finding the tool

You can find the latest version in PI package.

# Requirement

CPU: Strix and future programs.

BIOS: some functions may depend on specific BIOS. Those dependencies will be displayed on the help function of dll.

Operation System: Windows 10/11 64bit.

# Installation

Unzip the zip file.

# How to add a new dll

Each dll need to implement below five interfaces.

•	Support – No parameters. checks if the system supports this dll.

•	Help – No parameters. Print help message how to run the dll.

•	Init – 1 parameter. Initialize the dll with the parameter. It must be run at least once before “Run” function.

•	Run – No parameters. Run the dll with parameters defined in “Init”.

•	Stop – No parameters. Stop the dll from running.

You can get the sample dll source code on confluence page: https://confluence.amd.com/display/BIOSC/End+of+Manufacturer

EoM will call the functions of dll in a sequence like below:

# Driver

The dll runs in user mode and does not require signing. If the dll need to do something in kernel mode, a driver is required. We provide a common driver which has implemented most commonly used functions as below:

BOOL SendPspCmd(MBOX_COMMAND cmd, UINT8 *data, UINT32 inputSize, UINT32 outputSize, UINT32 socFamilyId, UINT8* result);

BOOL ReadIo32(UINT32 addr, UINT32* result);

BOOL WriteIo32(UINT32 addr, UINT32 value);

BOOL ReadIo8(UINT32 addr, UINT8* result);

BOOL WriteIo8(UINT32 addr, UINT8 value);

BOOL ReadPci32(UINT32 bus, UINT32 device, UINT32 function, UINT32 offset, UINT32* result);

BOOL WritePci32(UINT32 bus, UINT32 device, UINT32 function, UINT32 offset, UINT32 value);

BOOL ReadCpuId(UINT32 addr, UINT32* result);

BOOL GetSocFamilyId(UINT32* socFamilyId);

BOOL GetPspMmioBase(UINT32 socFamilyId, UINT32* mmioBase);

BOOL ApplyPhysicalMemory(UINT32 size, UINT64* va);

BOOL FreeMemory(UINT64 va);

BOOL ReadPhysicalMemory(UINT64 pa, UINT32 len, UINT8* data);

BOOL WritePhysicalMemory(UINT64 pa, UINT32 len, UINT8* data);

BOOL VaToPa(UINT64 va, UINT64* pa);

BOOL PaToVa(UINT64 pa, UINT64* va);

BOOL ReadMsr(UINT32 addr, UINT64* result);

BOOL WriteMsr(UINT32 addr, UINT64 value);

You can also check the sample “read PSB” code how to use those driver functions on confluence page.