# End of Manufacturing Tool User Manual

# About the tool

End of Manufacturing Tool (EoM) is designed to check and set system settings.

# Finding the tool

You can find the latest version in PI package.

# Requirement

CPU: Strix and future programs.

BIOS: some functions may depend on specific BIOS. Those dependencies will be displayed on the help function of dll.

Operation System: Windows 10/11 64bit. WinPE*.

*: To run the tool on WinPE, you need to manually download vcruntime140.dll from Microsoft official website and put it in the same folder of the tool. Reference link.

# Installation

Unzip the zip file.

# Usage

## Open command line

Open command line in Administrator mode. Then switch the command line to the tool folder.

## Modify config.json

Open config.json in text editor. EoM tool will provide a sample config.json file for reference. You need to modify this file to manage what functions you want to run and their sequence.

A sample config file may look like this:

{

"tools":

[

{"name":"ReadFar", "args":"-d -s"},

{"name":" WriteFar ", "args":"0 1"},

{"name":"Restart", "args":""},

{"name":" ReadFar ", "args":"-d"},

{"name":"Delay", "args":"1000"}

]

}

What you need to modify is the highlighted part. Each line contains two attributes:

- name: the name of the function

- args: the parameters of the function

With the two attributes, the tool will call the named function (given by the first attribute) and run the function with the parameter(given by the second attribute).

- Name

You can find all available function names in the same folder as EoM tool. Each file whose name ends with “.dll” is a valid name.

- args

The parameter for each function is different. If parameter is incorrect, tool will display help message how to run this function.

For details of args, please refer to section VI.

## Run EoM tool

EOM tool has two optional parameters(Only applies to EOM tool version >= 1.1.4.0):

- -c [configPath]: specify which config file you want to run. Default is “config.json”.

- -s [startFunction]: specify the index of function you want to run. Default is 0 (the first function).

After you have modified the config file, simply type “EoM.exe” and parameters in the command line you opened in step 1.

# Supported Functions and Args

Args in “<>” are optional. Args in “[]” are must.

| Function Name | Args | Description |
| --- | --- | --- |
| ReadAimt | <-d> <-s> | See * and ** |
| ReadFar | <-d> <-s> | See * and ** |
| ReadFips | <-d> <-s> | See * and ** |
| ReadFuseStatus | <-d> <-s> | See * and ** |
| ReadPsb | <-d> <-s> | See * and ** |
| ReadRpmc | <-d> <-s> | See * and ** |
| Restart |  | No args |
| WriteEom |  | No args |
| WriteFar | [splValue] [splConfig] | Example: “0 1” |
| WritePsb |  | No args |
| WriteRpmc | [RPMC address] | Example: “1” |
| Delay | <sleep time in seconds> | Example: “10”. |
| ReadEom | <-d> <-s> | See * and ** |

*-d: Debug mode. If a function runs in debug mode, it will print more debug messages. Otherwise, it will only return brief message, usually “PASS” or “FAIL”. Default is disabled. Only applies to EOM tool version >= 1.1.3.0.

**-s: Stop mode. If a function runs in stop mode, it will return error code and stop running whenever it encounters a failure. Otherwise, it will always return success. You can get the error code by %ERRORLEVEL%. Default is disabled. Only applies to EOM tool version >= 1.1.4.0.

# Function Details

## ReadAimt

Read system firmware table “RSMB” and display following SMBIOS table info:

- Wired Manageability Status

- Wireless Manageability Status

- AIM-T Implementation in BIOS

- AIM-T Factory APCB Status

- Wired Manageability Option

- Wireless Manageability Option

- AIM-T Option

- AIM-T State (Provisioned)

## ReadFar

Display if FAR is enabled or not. If FAR is enabled, it will also display if FAR is in develop mode or production mode.

If “-d” is specified, it will also display following info:

- HSTI status

- SPL status

- SPL fuse value

## ReadFips

Display if FIPS mode is on or off.

If “-d” is specified, it will also display following info:

- If FIPS provided approved algorithm service

## ReadFuseStatus

Display following info:

- Platform Secure Boot

- Root Key info

- Platform Vendor Id

- Platform Model Id

- Bios Key Revision Id

- Bios Key Anti-Rollback

- AMD WhiteList

- Secure Unlock

- Custom Fuse Region

## ReadPsb

Display if PSB check pass or fail.

If “-d” is specified, it will also display following info:

- PSB status

## ReadRpmc

Display if RPMC is enabled or not. If RPMC is enabled. It will also display which slot is in use.

If “-d” is specified, it will also display following info:

- SPI-ROM RPMC counter address 0~3 is available or not

- PSP/fTPM RPMC counter address 0~3 is in use or not

- ASIC RPMC fuse slot 0~15 is available or not

- RPMC Test Root Key is provisioned or not

- RPMC SPIROM is available or not

## Restart

Restart this computer. Tool will continue to run after restart, starting from the next command in config file.

## WriteEom

Write EoM fuse. Need reboot to take effect.

Once EoM fuse is written, you can no longer write FAR, PSB or RPMC fuse settings. We suggest you write this fuse as the last step of fuse settings.

## WriteFar

Write FAR(Firmware Anti-Rollback) fuse. Need reboot to take effect.

It has two parameters: SPL value and SPL config.

### SPL value = 0 and SPL config = 0

PSP FW will update SPL fuse from SPL table.

### SPL value > 0 and SPL config = 0

If CPU is not enforced, PSP will enforce FAR, set CPU SPL fuse to SPL value and will reject any further command.

### SPL config = 1

If CPU is not enforced, PSP will enforce FAR, set CPU SPL fuse to SPL value and will reject any further command.

Recommended setting for development phase:

SPL value = 0, SPL config = 1

This config allows FAR is enabled with CPU fused as 0.

Recommended setting for production phase:

SPL value = 0, SPL config = 0

This config allows FAR is enabled with CPU fused as SPL table value.

Please note that FAR switch is a one-time action.

You still need to turn on “SPL update switch” in AMD CBS options to follow SPL fuse upgrade.

## WritePsb

Write PSB fuse to enable Platform Secure Boot. Need reboot to take effect.

## WriteRpmc

Write RPMC fuse to enable Replay Protected Monotonic Counter. Need reboot to take effect.

It has one parameter: RPMC address

Tool will set RPMC address to the input address.

## Delay

Tool will sleep for a few seconds then continue running. User may press any key at any time to continue running immediately.

It has one parameter: Sleep time in seconds

If sleep time is not specified or input is 0 or not a number, tool will sleep for 7 days.

## ReadEom

Display if EoM fuse is enabled or not.

If “-d” is specified, it will also display following info:

- HSTI status

# Error Code

This section only applies to EOM tool version >= 1.1.3.0.

EOM error code is a 32-bit number. definition:

| Bit Field | Definition |
| --- | --- |
| [0:15] | Function postcode |
| [16:23] | Function code |
| [24:31] | Reserved, always 0 |

Each function has its own function code, specified in below table:

| Function Name | Function Code |
| --- | --- |
| ReadAimt | 0x1 |
| ReadFar | 0x2 |
| ReadFips | 0x3 |
| ReadFuseStatus | 0x4 |
| ReadPsb | 0x5 |
| ReadRpmc | 0x6 |
| Restart | 0x7 |
| WriteEom | 0x8 |
| WriteFar | 0x9 |
| WritePsb | 0xA |
| WriteRpmc | 0xB |
| Delay | 0xC |
| ReadEom | 0xD |

Each function has its own postcode definition, specified in below table:

(postcode 0 and 1 are common for all functions)

| Function Name | Postcode | Description |
| --- | --- | --- |
| * | 0 | Success |
| * | 1 | ERROR_CODE_PSP_COMMON_FAIL |
| ReadFar | 2 | ERROR_CODE_READ_HSTI_FAIL |
| ReadFar | 3 | ERROR_CODE_READ_PSB_FAIL |
| ReadFar | 4 | ERROR_CODE_READ_SPL_FUSE_FAIL |
| ReadFar | 5 | ERROR_CODE_FAR_NOT_ENABLE |
| ReadFips | 2 | ERROR_CODE_READ_HSTI_FAIL |
| ReadFips | 3 | ERROR_CODE_FIPS_NOT_ENABLE |
| ReadPsb | 2 | ERROR_CODE_PSB_NOT_ENABLED |
| ReadRpmc | 2 | ERROR_CODE_READ_RPMC_ENABLE_FAIL |
| ReadRpmc | 3 | ERROR_CODE_RPMC_NOT_ENABLE |
| WriteEom | 2 | ERROR_CODE_WRITE_EOM_FAIL |
| WriteFar | 2 | ERROR_CODE_SPL_CONFIG_ERROR |
| WriteFar | 3 | ERROR_CODE_FAR_ALREADY_ENABLED |
| WriteFar | 4 | ERROR_CODE_GET_FAR_STATUS_FAIL |
| WriteFar | 5 | ERROR_CODE_TABLE_NOT_PRESENT |
| WriteFar | 6 | ERROR_CODE_ENTRY_NOT_PRESENT |
| WriteFar | 7 | ERROR_CODE_SPL_VALUE_INVALID |
| WriteFar | 8 | ERROR_CODE_SPL_FUSE_UPDATE_REQUIRED |
| WriteFar | 9 | ERROR_CODE_WRITE_FAR_FAIL |
| WritePsb | 2 | ERROR_CODE_SET_PSB_FUSE_FAIL |
| WriteRpmc | 2 | ERROR_CODE_WRITE_RPMC_FAIL |
| WriteRpmc | 3 | ERROR_CODE_INPUT_PARAM_INVALID |
| ReadEom | 2 | ERROR_CODE_READ_HSTI_FAIL |
| ReadEom | 3 | ERROR_CODE_EOM_NOT_ENABLE |

Example: If you get an EOM error code 0x30002, this means function code 3 and postcode 2. This indicates EOM tool fails in ReadFips, and error is ERROR_CODE_READ_HSTI_FAIL.

# FAQ

## It reports error “Missing ucrtbase.dll” or “Missing vcruntime140.dll”.

Some systems do not contain above dlls by default. You can search on website and download those missing dlls: Reference link. You may need to install the downloaded files. After installation, find those files and put them in the same folder of the tool. Then run the tool again.

# Copyright

AMD, the AMD Arrow logo, and combinations thereof, are trademarks of Advanced Micro Devices, Inc.

Product names used in this publication are for identification purposes only and may be trademarks of their respective companies.

Copyright (C) 2008-2024 Advanced Micro Devices, Inc. All rights reserved.