1.   Prepare a U disk（USB drive）  with a storage capacity of at least 16 GB

2. Right click and select format... Format U disk to FAT32 format,and change the

Volume label to WINPE ,click start to finish formatting

- Copy your system files to U disk, and it must be in the root directory of U disk.

4. Restart the computer, press F7 repeatedly on  the boot logo interface, and select

USB boot in the boot options

5.Type fs3:(you may type fs1-fs4,depends on where the USB drive is ) , then press enter

6.Type dir or ls , press enter so that you can locate the files

7.Type start, press tab,then press enter.

8.Start flashing, if there is a similar reminder, enter exit.