1.   Prepare a USB disk（USB drive）

2. Right click and select format... Format USB disk to FAT32 format,and change the

Volume label to WINPE ,click start to finish formatting

- Copy your system files to USB disk, and it must be in the root directory of USB disk.

4. Restart the computer, press F7 repeatedly on  the boot logo interface, and select

USB boot in the boot options

5.If the BIOS flashes automatically, just wait till it finishes. If not, type fs3:(you may type fs1-fs4,depends on where the USB drive is ) , then press enter

6.Type dir or ls , press enter so that you can locate the files

7.Type f.nsh, press tab,then press enter.

8.Start flashing, if there is a similar reminder, enter exit.