1.   Prepare a USB disk（USB drive）  with a storage capacity of at least 4 GB

2. Right click and select format... Format U disk to FAT32 format,and change the

Volume label to WINPE ,click start to finish formatting

- Copy the BIOS file to USB disk, and it must be in the root directory of USB disk.

4. Restart the computer, press F7 repeatedly on  the boot logo interface, and select

USB boot in the boot options

5.Type fs3:(you may type fs0:-fs4:,depends on where the USB drive is ) , then press enter

6.Type dir or ls , press enter so that you can locate the files

7.Type flash, press tab,then press enter.

- 8.APTl0 SETUP.AMl screen pop up,please select PROCEED WITH FASH UPDATE.

Start flashing, if there is a similar reminder, enter exit.